# Facilities Checklist for the Rainy Season

## How to Use This Checklist

- Complete the first pass before the first major storm. El Niño effects are most likely during winter 2026-27, and timing and severity vary by region, so check your local National Weather Service forecast.

- Assign an owner and a due date to every item.

- Repeat the walk after the first rain and after every major storm.

- Add items that apply to your campus. A campus on a slope has different needs from one in a low-lying area.

## Checklist

| Area | Task | Owner | Due date | Status and notes |
| --- | --- | --- | --- | --- |
| Drainage | Clean gutters, drains, and downspouts. Confirm that downspouts carry water away from buildings. |  |  |  |
| Drainage | Check storm drains, catch basins, and low spots in parking lots, play areas, and walkways. |  |  |  |
| Roof and buildings | Inspect the roof for damage, ponding, and known leaks. Repair before the rainy season. |  |  |  |
| Roof and buildings | Check windows, doors, skylights, and seals for leaks. Mark rooms with a history of leaks. |  |  |  |
| Grading | Check grading and drainage on campus. Note where water collects or flows toward buildings. |  |  |  |
| Slopes and retaining walls | Check hillsides and retaining walls for erosion, cracks, or signs of movement. Identify areas where mud or debris could reach students. |  |  |  |
| Sandbags | Stock sandbags and know where to get more. Identify doors and low points where they would be placed. |  |  |  |
| Trees | Trim trees and remove dead branches near buildings, power lines, walkways, and play areas. |  |  |  |
| Outdoor equipment | Secure or store outdoor furniture, trash cans, signs, and play equipment that can blow around. |  |  |  |
| Power | Test backup power and emergency lighting. Stock flashlights and batteries. |  |  |  |
| Power | Identify which systems stop working in an outage: alarms, locks, gates, PA, phones, internet. Make a workaround for each. |  |  |  |
| Utilities | Know the location of the water and electrical shutoffs. Label them and name who is trained to use them. |  |  |  |
| Communications | Test radios and backup communication tools. Confirm charged batteries. |  |  |  |
| Access and egress | Check which entrances, paths, and parking lots flood first. Note alternate routes. |  |  |  |
| Shelter areas | Identify upper-floor or interior shelter areas away from water and windows. Confirm capacity. |  |  |  |
| Supplies | Stock water, first aid supplies, blankets, towels, trash bags, and cleanup supplies. |  |  |  |
| Records and equipment | Move critical records and electronics off floors and away from windows and leak-prone areas. |  |  |  |
| Vendors | List contacts for roofing, plumbing, electrical, water extraction, and cleanup vendors. Confirm after-hours numbers. |  |  |  |
| Insurance | Confirm coverage and the claims process. Know where policies are stored, including offline copies. |  |  |  |

## Walk the campus after the first rain

- Record where water pooled, entered buildings, or flowed in the wrong direction.

- Check for new roof leaks and wet ceiling tiles.

- Check drains for blockages and debris.

- Check slopes, trees, and fences for damage.

- Take dated photos of every problem.

- Send a work request for each item with an owner and a date.

## Damage record

| Date | Location | Description | Photo taken | Work request sent | Insurer notified |
| --- | --- | --- | --- | --- | --- |

## Key contacts

| Role | Name | Phone | After-hours number |
| --- | --- | --- | --- |
| Facilities lead | [Name] | [Phone] | [Phone] |
| Utility company | [Name] | [Phone] | [Phone] |
| Roofing | [Name] | [Phone] | [Phone] |
| Water extraction and cleanup | [Name] | [Phone] | [Phone] |
| Insurance | [Name] | [Phone] | [Phone] |
| District, network, or school office | [Name] | [Phone] | [Phone] |